ME4027 COMPUTATIONAL FLUID DYNAMICS

COURSE PROJECT

MONSOON 2023-24

ANSYS FLUENT PROJECT

Submitted by
THANVIR DIOUF S 
B201120ME
S7 ME03

PROBLEM STATEMENT

Consider two horizontal plates separated by a distance of 20 mm. The length of the plates is 500 mm each. Water is flowing in between them at a velocity of 5 m/s. Consider the following two cases:

a) When the two plates are stationary,

b) When the top plate is given a horizontal velocity of 5 m/s.

Find the velocity distribution at 𝑥 = 175 mm. Find the 𝑦 −distance from the bottom plate where the velocity is maximum at that location.

Compare the results of (a) and (b).

INITIAL BOUNDARY CONDITIONS

- Inlet pressure is zero

- Outlet pressure is zero

- Temperature is uniform

- Bottom plate is fixed

- Water density is 997 kg/m3

- Water viscosity is 0.00089 Pa s

- Top plate is:

- Fixed in case 1

- Has 5 m/s velocity in x-direction in case 2

GEOMETRY CREATION AND MESHING STEPS

Made the geometry in Ansys 2023 students edition using the design modeler.

Dimensions are 500mm x 20mm.

Meshing is also done in Ansys.

Element size is 0.5 mm.

Magnified view

After meshing

PROBLEM SETUP

- Start Ansys Workbench and load Fluent. Click on Geometry and open using Design Modeler.

- Draw a rectangle on the XY plane of dimension 500mm x 20mm.

- Now close the design modeler and edit the mesh. Set the element size to 0.5 mm and click on generate. Then name the sides of the rectangle (inlet, outlet, upper_wall and lower_wall). Click generate and exit mesh editor.

- Open setup and edit the material and set density and viscosity to that of water.

- Set the boundary conditions. Set inlet and outlet pressure as 0. In second case set upper_wall velocity to 5 m/s.

- Right-click on Initialization and click initialize. Then run calculations. Set number of iterations to some number. I set it to 1000. Since this is a simple calculation it happens under 2 minutes.

- Double click on XY Plot under Results. Create a new line at x=175mm and then plot the Y position vs x-velocity graph.

- Click on results and the generate the contour plots, vector plots, stream lines, path lines and profile.

RESIDUAL HISTORY

POST PROCESSING AND RESULTS

Contour plots:

Case 2

Case 1

Pathlines:

Case1

Case 2

Plotlines:

Case 1

Case 2

Streamlines:

Case 1

Case 2

Vector Plots:

Case 1

Case 2

CONCLUSION

In this project, we have conducted a study about the velocity distribution of water flowing between 2 plates. The simulation results are the same as the expected output aligning with our knowledge of 2-Dimensional fluid flow. This study is important as the data can be used for situations where we use water as a lubricant between 2 flat surfaces.